BusLogic QuickTech Guides

Multiple SCSI Controllers

In order to integrate multiple BusLogic SCSI Controllers in your PC, do the following steps:

1) Each card must have a Unique and FREE Interrupt (IRQ), DMA Channel, and port address.

2) Enable the BIOS on the Primary Controller, DISABLE the BIOS on all other controllers.

3) Insert all controllers into the system, connect all devices properly (See Manual).

4) Load the BusLogic Device Driver as many times as there are SCSI Controllers in the System, specify the port address uniquely for each load.