BusLogic QuickTech Guides

IDE Products

The purpose for this document is to act as an installation checklist to properly install BusLogic IDE Host Adapters into your system. This document will proceed with explanation of each step.  Let’s begin the installation with system requirements.

STEP 0 - REQUIREMENTS

You will need the following to properly install your BusLogic IDE Host Adapter into your system:

1) BusLogic IDE Host Adapter with current Firmware and BIOS

2) Motherboard with current BIOS  (call motherboard manufacturer for current

version). The motherboard must  support a bus type for your desired card

(PCI, VL-Bus, ISA).

3) BusLogic Distribution Diskettes and Manual(s) when applicable

4) A Phillips screwdriver

STEP 1 - ESTABLISH YOUR SYSTEM SETTINGS

Before you configure your adapter for installation, you need to know some attributes of your current hardware and software setup. Let’s begin by establishing a free slot:

ISA Boards 		Any 16-bit ISA Slot.

VL-BUS		A Free VL-Bus Slot

PCI 		           A Free PCI Slot

Now, let’s establish a free interrupt for your IDE Host Adapter. Valid values are 14 & 15.  The default IRQ for BusLogic IDE Adapter is 14. Also, you must make the particular IRQ free and not reserved (PCI) in some motherboards.

Finally, let’s establish a free BIOS location. The default is C800h. Our BIOS uses 16k. Manually inspect other cards for BIOS locations.

FREE BIOS Memory location is: _____________________________

STEP 2 - Setting the Card Jumpers and Installing

Now that you have established the settings for your card, you will need to configure the card before installing. To set the card settings do the following:

1) ISA 		Set the card to appropriate settings as per the manual.

2) VL-BUS		Set the card to appropriate settings as per the manual.

Make sure Bus Speed is set on your IDE Card

(See Manual) and your motherboard.

3) PCI		Software Configurable. Jumpers present for non

compliant PCI motherboards. Install Paddleboard

if you wish to use IRQ 14.

Now that the card is set properly, firmly and evenly insert the card into the previously selected slot. Screw in the bolting screw to firmly secure the board in place.

Note: Make sure that the VL-BUS IDE Host Adapter is firmly installed,

and that one side does not slip out when inserting the other side.

Turn the machine on, then when the BusLogic Banner comes up, press F2 or F6 to go into the BusLogic AutoIDE BIOS. There, you can set any software settings like Disk Attributes and Mirroring or Linking.

Cable Selection		Only use high grade, pre-tested cabling. The total cable 				length may not exceed 18”.

Note: Always try to minimize the IDE cable						length. The shorter the cable, the more stable the bus.

It is recommended that the end of the internal cable 					always be used.

RAM				You need to have at least 512KB of RAM for the 					board to run. We recommend 4MB of RAM

STEP 3 - Configure Your Drives

In order to properly configure 2 hard drives on your BusLogic IDE Host Adapter, you need to set jumpers on both drives. You need to set the primary boot drive as Master and the second drive as Slave. You can get the settings for your drive from the Drive Manufacturer.

Note: Not all combinations of Master Slave settings work properly. If you are experiencing problems, try switching the order or Master/Slave. If that still doesn’t work, set both drives to master and place the second drive on the second IDE Channel (See Manual).

If you wish to install 3 or 4 drive in your system, you need to place the 3rd and

4th drive on channel 1 and Set 1 Drive as Master and the other as slave just like on channel 0.

STEP 4 - Connecting Everything and Getting Started

Now that everything is configured properly, turn off the power to your system, ground yourself and plug in all devices into the IDE cable and plug the cable chain into the IDE controller.  Make sure Pin 1 on the IDE cable matches Pin 1 on the Drive (See device documentation), make sure all 40 pins are tightly and evenly plugged in to the cable. Plug in the power cables to each of the drives. Make sure all connections are solid. Verify that in your CMOS setup, all IDE Drives are  Installed as a Drive Type 1 (Both if 2 Drive, >2 drives doesn’t matter).  Then, turn on the system and Press F2 or F6 to go into AutoIDE.

STEP 5 - Configuring Drivers and Hard Drives

When in AutoIDE, proceed to the Disk Parameter Setup. Verify that all drives are being seen by our controller. If you have an incompatible mapping scheme on your drive from a previous format, you may need  to perform a Low Level Format from our controller from AutoIDE’s main menu. Once established, then we are ready to partition and format the drives.

Boot off of a DOS boot disk and run DOS FDISK to partition the drive.

Then, type:

A:>FORMAT C:/S/V

After the format, reboot and boot off of the newly partitioned hard drive. Proceed to partition and format all new drives in your system that need to be prepared.

To Support the Following:

ATAPI and CD-ROM:

BusLogic IDE Host adapters do not currently support ATAPI or CD-ROM Drives

Windows(tm) 32-Bit Disk and File Access

Call BusLogic’s BBS and download the file WINIDE2.EXE, decompress the files to a sub-directory, and run the included SETUP program in Windows. If you have

Windows(tm) for Workgroups, go into the 386 Enhanced Control Panel and under Advanced Settings, turn 32-Bit File Access On.

Note: The most current version of our Device Driver can be found on 	  BusLogic’s Tech Support BBS at (408) 492-1984  14.4Kbps,8,n,1.

Mirroring

You need to put 2 drives of similar size on each channel on the IDE Host Adapter, then go into Disk Parameter setup and change the Attribute of D0 to

[D0 mirror D1]. Now all operations on D0 will be mirrored back to D1.

Note: This is destructive to data on the Mirror drive, so back up data and then

restore it to the primary drive. Also, your Mirrored drive should be >=

the Primary Drive.

Linking

Place both drive on Channel 0 and turn on the attribute [D0 link D1], they will now act like one drive.

Note: This is destructive to data on the drive, so back up data and then restore it.

Drives >528MB

In Disk Parameter Setup, Enable Trk-Remap for all drives >528MB.

Note: You will need to recreate the disk partition by deleting all partitions and recreating them. This is destructive to data on the drive, so back up data and then restore it.

(c) 1994 BusLogic Corporation, No portion of this document may be reproduced without permission by BusLogic