An

Introduction

to

IGRP

Charles L. Hedrick

RUTGERS

The State University of New Jersey

Center for Computers and Information Services

Laboratory for Computer Science Research

22 August 1991

Acknowledgements

This  paper  really  should  show Len Bosack of cisco Systems as co-author, and

possibly should also list an unidentified  lawyer  at  Townsend  and  Townsend.

Most of the ideas behind IGRP are Len's.  However since he has't even seen this

paper, it would seem misleading to list him as an author.

This paper is based on the patent application for IGRP, and on  an  examination

of  the  code implementing IGRP in the cisco gateways.  Unlike previous drafts,

this paper follows the code more closely than  the  patent  application.    The

exceptions  are  a  couple of features which are not yet implemented, but which

are planned to be.  The background and evaluative comments are my own.  However

when  it  came  to  giving  equations and detailed algorithmic descriptions, it

seemed to make more sense simply to lift them from the patent  application,  to

the  extent  that they were consistent with the way the code actually works.  I

have tried to translate them from legalese into wording  more  appropriate  for

technical  discussions,  but  inevitably  there are going to be similarities in

both organization and wording between this paper and  the  patent  application.

Unfortunately  patent  applications  do  not  follow  the  usual conventions of

scholarly attribution.  But it is quite clear that most of the ideas in it came

from  Len  Bosack, with some wording contributed by Townsend and Townsend.  But

of course I am responsible for the final form, and should bear  the  blame  for

any errors that may appear.

1

This  paper  is  an  introduction to IGRP, cisco System's Inter-Gateway Routing

Protocol.  It has two purposes.  One is to form an  introduction  to  the  IGRP

technology,  for  those  who  are interested in using, evaluating, and possibly

implementing it.  The other is to give wider exposure to some interesting ideas

that  are  embodied  in  IGRP.   I believe even those who are not interested in

using IGRP per se may find some of these ideas useful.

This draft is intended for discussion in the open distance-vector group  within

the IETF.  Thus some attempt has been made to give protocol details.  It is not

clear whether there are enough details to do a complete implementation or  not.

However that is the intention.

This  document  has  been checked carefully against the source code for cisco's

Gateway Server, release 8.0(3).  Where possible, the description is  consistent

with  that  code.    However  no  attempt  has been made to describe all of the

internal data structures or algorithms, since this is a  protocol  description,

not an implementation design.  In particular, here are some areas in which this

description can be expected to differ from cisco's implementation:

- cisco does not yet implement multiple types of  service,  nor  tests  that

depend  upon  the  hop  count.  However they do maintain and propagate the

necessary information.  The description here indicates how I believe  that

information will be used in future releases.

- cisco  has  a  number  of  administrative  controls,  allowing filters and

modifications of various sorts on routing information.  Since these  apply

to all routing protocols implemented by cisco, I have chosen to view these

controls as features of the implementation, and not part of the  protocol.

They  are  not  reflected  in  any of the data exchanged between gateways.

Thus  there  is  nothing  to  prevent  other  implementors  from  choosing

different  sorts of controls.  In previous releases of the cisco software,

"administrative distance" appeared to be part of IGRP.   However  in  8.0,

administrative  distance is not propagated by IGRP, but is purely local to

the gateway.  I do not consider it be part of the  protocol,  and  do  not

describe it here.

- cisco  supplies  several  ways  to  define  default routes.  Again, I have

chosen to regard this as a feature of the implementation rather  than  the

protocol.    However  the concept of "exterior routes" is reflected in the

protocol, and will be described here.

This edition has been  updated  based  on  Cisco's  release  8.2(5).    A  full

comparison  against  the  source  code has not been done.  However changes have

been made to reflect the fact that variance is no longer implemented, and  that

holddowns may now be disabled.

The  technology described here is the subject of a patent application by cisco.

This  paper  is  being  circulated  with  cisco's  permission.    However  that

permission  should  not  be construed as giving any sort of license.  The legal

implications of implementing the ideas described in this paper are between you,

your lawyer, and cisco.

1. Goals for IGRP

IGRP  is  a  protocol  that  allows  a  number  of gateways to coordinate their

routing.  Its goals are

- stable routing even in very large or complex networks.  No  routing  loops

2

should occur, even as transients.

- fast response to changes in network topology

- low  overhead.    That  is, IGRP itself should not use more bandwidth than

what is actually needed for its task.

- splitting traffic among several parallel routes when they are  of  roughly

equal desirability.

- taking into account error rates and level of traffic on different paths

- the  ability  to  handle  multiple "types of service" with a single set of

information.

The current implemention of IGRP handles routing for TCP/IP.  However the basic

design is intended to be able to handle a variety of protocols.

During the last few years, routing has suddenly become a more difficult problem

than it used to be.  A few years ago, protocols such as RIP were sufficient  to

handle   most   real   networks.      However   growth  in  the  Internet,  and

decentralization of control of its structure, have now resulted in a system  of

networks  that is nearly beyond our capabilities to manage.  Similar situations

are occuring in large corporate networks as well.  IGRP is one tool intended to

help attack this problem.

No one tool is going to solve all routing problems.  Conventionally the routing

problem is broken into several pieces.   Protocols  such  as  IGRP  are  called

"internal  gateway  protocols"  (IGP's).    They  are intended for use within a

single set of networks, either under a single management or closely coordinated

managements.    Such  sets  of  networks  are  connected  by  "external gateway

protocols" (EGP's).  An IGP is designed to keep track of a good deal of  detail

about  network  topology.   Priority in designing an IGP is placed on producing

optimal routes and responding quickly to  changes.    An  EGP  is  intended  to

protect  one system of networks against errors or intentional misrepresentation

by  other  systems.    Priority  in  designing  an  EGP  is  on  stability  and

administrative  controls.    Often  it  is  sufficient  for an EGP to produce a

reasonable route, rather than the optimal route.  In fact, there  are  features

in  cisco's  implementation  that  allow  IGRP  to  be  used  as an EGP in some

circumstances.  However the emphasis in its design is on use as an IGP.

IGRP  has  some  similarities  to  older  protocols  such  as  Xerox's  Routing

Information  Protocol,  Berkeley's RIP, and Dave Mills' Hello.  It differs from

these protocols primilarly in  being  designed  for  larger  and  more  complex

networks.    Section  4 gives a more detailed comparison with RIP, which is the

most widely used of the older generation of protocols.

Like these older protocols, IGRP is a distance vector  protocol.    In  such  a

protocol,  gateways  exchange  routing information only with adjacent gateways.

This routing information contains a summary of information about  the  rest  of

the  network.    It  can be shown mathematically that all of the gateways taken

together are solving an optimization problem by what amounts to  a  distributed

algorithm.    Each gateway only needs to solve part of the problem, and it only

has to receive a portion of the total data.

The major alterative is a class of algorithms referred to as SPF (shortest-path

first).    These are based on a flooding technique, where every gateway is kept

up to date about the status of every interface on every other  gateway.    Each

gateway  independently  solves  the optimization problem from its point of view

using data for the entire network.  There are advantages to each approach.   In

some  circumstances  SPF  may  be  able to respond to changes more quickly.  In

3

order to prevent routing loops, IGRP has to ignore new data for a  few  minutes

after certain kinds of changes.  Because SPF has information directly from each

gateway, it is able to avoid these routing loops.   Thus  it  can  act  on  new

information  immediately.  However SPF has to deal with substantially more data

than IGRP, both in internal data structures and in messages  between  gateways.

Thus  SPF  implementations  can  be  expected to have higher overhead than IGRP

implementations, other things being equal.

2. The Routing Problem

IGRP is intended for use in gateways connecting several networks.    We  assume

that  the  networks use packet-based technology.  In effect the gateways act as

packet switches.  When a system connected to one network wants to send a packet

to  a  system on a different network, it addresses the packet to a gateway.  If

the destination is on one of the networks connected to the gateway, the gateway

will  forward  the  packet  to  the  destination.    If the destination is more

distant, the gateway will forward the packet to another gateway that is  closer

to the destination.  Gateways use routing tables to help them decide what to do

with packets.  Here is a simple example routing table.  (Addresses used in  the

examples  are  IP addresses taken from Rutgers University.  Note that the basic

routing problem is similar for other protocols as well,  but  this  description

will assume that IGRP is being used for routing IP.)

network    gateway     interface

128.6.4    none        ethernet 0

128.6.5    none        ethernet 1

128.6.21   128.6.4.1   ethernet 0

128.121    128.6.5.4   ethernet 1

10         128.6.5.4   ethernet 1

(Actual IGRP routing tables have additional information for each gateway, as we

will see.) This gateway is connected to two Ethernets, called 0 and  1.    They

have  been  given  IP  network  numbers  (actually  subnet numbers) 128.6.4 and

128.6.5.  Thus packets addressed  for  these  specific  networks  can  be  sent

directly   to  the  destination,  simply  by  using  the  appropriate  Ethernet

interface.  There are two nearby gateways, 128.6.4.1 and  128.6.5.4.    Packets

for  networks  other  than  128.6.4 and 128.6.5 will be forwarded to one or the

other of those gateways.  The routing table indicates which gateway  should  be

used for which network.  For example, packets addressed to a host on network 10

should be forwarded to gateway 128.6.5.4.   One  hopes  that  this  gateway  is

closer  to network 10, i.e.  that the best path to network 10 goes through this

gateway.  The primary purpose of IGRP  is  allow  the  gateways  to  build  and

maintain routing tables like this.

3. Summary of IGRP

As  mentioned  above, IGRP is a protocol that allows gateways to build up their

routing table by exchanging information with other gateways.  A gateway  starts

out with entries for all of the networks that are directly connected to it.  It

gets information about  other  networks  by  exchanging  routing  updates  with

adjacent  gateways.   In the simplest case, the gateway will find one path that

represents the best way to get to each network.  A path is characterized by the

next gateway to which packets should be sent, the network interface that should

be used, and metric information.  Metric information is a set of  numbers  that

4

characterize  how  good  the path is.  This allows the gateway to compare paths

that it has heard from various gateways and decide which one to use.  There are

often  cases  where  it makes sense to split traffic between two or more paths.

IGRP will do this whenever two or more paths are equally good.   The  user  can

also configure it to split traffic when paths are almost equally good.  In this

case more traffic will be sent along the path with  the  better  metric.    The

intent  is  that  traffic  can be split between a 9600 bps line and a 19200 bps

line, and the 19200 line will get roughly twice as much traffic as the 9600 bps

line.

The metric used by IGRP includes

- the topological delay time

- the bandwidth of the narrowest bandwidth segment of the path

- the channel occupancy of the path

- the reliability of the path.

Topological  delay  time  is  the  amount  of  time it would take to get to the

destination along that path, assuming an unloaded network.  Of course there  is

additional  delay when the network is loaded.  However load is accounted for by

using the channel occupany figure, not by attempting to measure actual  delays.

The  path  bandwidth  is simply the bandwidth in bits per second of the slowest

link in the path.  Channel occupancy indicates how much of  that  bandwidth  is

currently  in  use.    It  is measured, and will change with load.  Reliability

indicates the current error rate.  It is the fraction of packets that arrive at

the destination undamaged.  It is measured.

Although  they  are  not  used  as  part  of the metric, two addition pieces of

information are passed with it: hop count and MTU.  The hop count is simply the

number  of  gateways  that  a  packet  will  have  to  go through to get to the

destination.  MTU is the maximum packet size that can be sent along the  entire

path  without  fragmentation.   (That is, it is the minimum of the MTU's of all

the networks involved in the path.)

Based on the metric information, a single "composite metric" is calculated  for

the  path.    The  composite  metric  combines the effect of the various metric

components into a single number representing the "goodness" of that path.    It

is the composite metric that is actually used to decide on the best path.

Periodically  each  gateway  broadcasts  its  entire  routing  table (with some

censoring because of the split horizon rule) to all adjacent gateways.  When  a

gateway  gets  this  broadcast from another gateway, it compares the table with

its existing table.  Any new destinations and paths are added to the  gateway's

routing  table.  Paths in the broadcast are compared with existing paths.  If a

new path is better, it may replace  the  existing  one.    Information  in  the

broadcast  is also used to update channel occupancy and other information about

existing paths.  This general procedure is similar to that used by all distance

vector  protocols.    It  is  referred to in the mathematical literature as the

Bellman-Ford algorithm.  For a detailed development of the basic procedure, see

RFC 1058, which describes RIP, an older distance vector protocol.

In  IGRP,  the  general  Bellman-Ford  algorithm  is modified in three critical

aspects.  First, instead of a simple metric, a vector of  metrics  is  used  to

characterize paths.  Second, instead of picking a single path with the smallest

metric, traffic is split  among  several  paths,  whose  metrics  fall  into  a

specified  range.   Third, several features are introduced to provide stability

in situations where the topology is changing.

5

The best path is selected based on a composite metric

[(K1 / Be) + (K2 * Dc)] r

Where:

K1, K2 = constants;

Be = unloaded path bandwidth x (1 - channel occupancy)

Dc = topological delay; and

r = reliability.

The path having the smallest composite metric will be the  best  path.    Where

there  are  multiple  paths  to the same destination, the gateway can route the

packets over more than one path.  This is done in accordance with the composite

metric for each data path.  For instance, if one path has a composite metric of

1 and another path has a composite metric of 3, three  times  as  many  packets

will  be  sent  over  the data path having the composite metric of 1.  However,

only paths whose composite metrics are with a certain  range  of  the  smallest

composite metric will be used.  K1 and K2 indicate the weight to be assigned to

bandwidth and delay.  These will depend  upon  the  "type  of  service".    For

example, interactive traffic would normally place a higher weight on delay, and

file transfer on bandwidth.

There are two advantages to using a vector of metric information.  The first is

that it provides the ability to support multiple types of service from the same

set of data.  The second advantage is improved accuracy.  When a single  metric

is  used,  it is normally treated as if it were a delay.  Each link in the path

is added to the total metric.  If there is a link with a low bandwidth,  it  is

normally  represented  by  a  large delay.  However bandwidth limitations don't

really cumulate the way delays  do.    By  treating  bandwidth  as  a  separate

component,  it  can  be handled correctly.  Similarly, load can be handled by a

separate channel occupancy number.

IGRP provides a system for interconnecting computer networks which  can  stably

handle  a  general  graph  topology including loops.  The system maintains full

path metric information, i.e., it  knows  the  path  parameters  to  all  other

networks  to  which  any gateway is connected.  Traffic can be distributed over

parallel paths and multiple path parameters can be simultaneously computed over

the entire network.

4. Comparison with RIP

This  section compares IGRP with RIP.  This comparison is useful because RIP is

used widely for purposes similar to IGRP.  However doing this is  not  entirely

fair.    RIP  was  not intended to meet all of the same goals as IGRP.  RIP was

intended for use in small networks with reasonably uniform technology.  In such

applications it is generally adequate.

The  most  basic  difference  between  IGRP  and  RIP is the structure of their

metrics.  Unfortunately this is not a change that can simply be retrofitted  to

RIP.  It requires the new algorithms and data structures present in IGRP.

RIP  uses  a  simple  "hop count" metric to describe the network.  Unlike IGRP,

where every path is described by  a  delay,  bandwidth,  etc.,  in  RIP  it  is

described  by a number from 1 to 15.  Normally this number is used to represent

how many gateways the path goes through  before  getting  to  the  destination.

This  means  that  no  distinction  is  made  between a slow serial line and an

6

Ethernet.  In some implementations of  RIP,  it  is  possible  for  the  system

administrator  to  specify  that  a given hop should be counted more than once.

Slow networks can be represented by a large hop count.  But since  the  maximum

is  15, this can't be done very much.  E.g.  if an Ethernet is represented by 1

and a 56Kb line by 3, there can be at most 5 56Kb  lines  in  a  path,  or  the

maximum  of  15 is exceeded.  In order to represent the full range of available

network speeds, and allow for a large network, studies done  by  cisco  suggest

that a 24-bit metric is needed.  If the maximum metric is too small, the system

administrator  is  presented  with  an  unpleasant  choice:  either  he   can't

distinguish  between  fast  and  slow routes, or he can't fit his whole network

into the limit.  In fact a number of national networks  are  now  large  enough

that  RIP can't handle them even if every hop is counted only once.  RIP simply

can't be used for such networks.

The obvious response  would  be  to  modify  RIP  to  allow  a  larger  metric.

Unfortunately,  this  won't  work.  Like all distance vector protocols, RIP has

the problem of "counting to infinity".  This is described in more detail in RFC

1058.   When topology changes, spurious routes will be introduced.  The metrics

associated with these spurious routes slowly increase until they reach  15,  at

which  point  the  routes  are removed.  15 is a small enough maximum that this

process will converge fairly quickly, assuming that triggered updates are used.

If  RIP were modified to allow a 24-bit metric, loops would persist long enough

for the metric to be counted up to 2**24.  This is not  tolerable.    IGRP  has

features  designed to prevent spurious routes from being introduced.  These are

discussed below in section 5.2.  It is not practical to handle complex networks

without introducing such features or changing to a protocol such as SPF.

IGRP  does  a bit more than simply increase the range of allowable metrics.  It

restructures the metric to describe delay, bandwidth,  reliability,  and  load.

It  is  possible  to  represent  such considerations in a single metric such as

RIP's.  However the approach taken by IGRP is potentially more accurate.    For

example,  with a single metric, several successive fast links will appear to be

equivalent to a single slow one.  This may be the case for interactive traffic,

where  delay  is  the  primary  concern.    However for bulk data transfer, the

primary concern is bandwidth, and adding metrics  together  is  not  the  right

approach  there.    IGRP  handles  delay  and  bandwidth separately, cumulating

delays, but taking the minimum of the bandwidths.  It is not easy to see how to

incorporate the effects of reliability and load into a single-component metric.

In  my opinion, one of the big advantages of IGRP is ease of configuration.  It

can directly represent quantities that have physical meaning.  This means  that

it can be set up automatically, based on interface type, line speed, etc.  With

a single-component metric, the metric is more likely to have to be "cooked"  to

incorporate effects of several different things.

Other  innovations  are more a matter of algorithms and data structures than of

the routing  protocol.    For  example,  IGRP  specifies  algorithms  and  data

structures  that  support  splitting  traffic  among  several  routes.    It is

certainly possible to design an implementation of RIP that does this.   However

once routing is being reimplemented, there is no reason to stick with RIP.

So  far  I  have  described  "generic  IGRP",  a technology which could support

routing for any network  protocol.    However  in  this  section  it  is  worth

mentioning  a  bit  more about the specific TCP/IP implementation.  That is the

implementation that is going to be compared with RIP.

RIP update messages simply contain snapshots of the routing table.    That  is,

they  have a number of destinations and metric values, and little else.  The IP

implementation of IGRP has additional structure.  First, the update message  is

identified by an "autonomous system number."  This terminology comes out of the

Arpanet tradition, and has specific meaning there.  However for  most  networks

7

what it means is that you can run several different routing systems on the same

network.  This is useful for places where networks from  several  organizations

converge.  Each organization can maintain its own routing.  Because each update

is labelled, gateways can be configured to pay attention only to the right one.

Certain  gateways  are  configured  to  receive updates from several autonomous

systems.  They pass information between the systems  in  a  controlled  manner.

Note that this is not a complete solution to problems of routing security.  Any

gateway can be configured to listen to  updates  from  any  autonomous  system.

However  it  is still a very useful tool in implementing routing policies where

is a reasonable degree of trust between the network administrators.

The second structural feature  about  IGRP  update  messages  affects  the  way

default  routes  are handled by IGRP.  Most routing protocols have a concept of

default route.  It is often not practical for routing  updates  to  list  every

network  in  the  world.    Typically  a  set of gateways need detailed routing

information  for  networks  within  their  organization.    All   traffic   for

destinations  outside  their  organization can be sent to one of a few boundary

gateways.  Those boundary gateways may have more  complete  information.    The

route to the best boundary gateway is a "default route".  It's a default in the

sense that it is used to get to any destination that is not listed specifically

in  the  internal  routing  updates.    RIP,  and some other routing protocols,

circulate information about the default route as if it  were  a  real  network.

IGRP  takes  a  different  approach.    Rather than a single fake entry for the

default route, IGRP allows real networks to be flagged as candidates for  being

a  default.  This is implemented by placing information about those networks in

a special exterior section of the update message.  However it might as well  be

thought  of  as  turning on a bit associated with those networks.  Periodically

IGRP scans all the candidate default routes and chooses the one with the lowest

metric as the actual default route.

Potentially  this  approach  to  defaults  is  somewhat  more flexible than the

approach taken by most RIP implementations.  Most typically RIP gateways can be

set to generate a default route with a certain specified metric.  The intention

is that this would be done at boundary gateways.  However there are  situations

where  this  isn't  really  good  enough.    Suppose that Rutgers University is

directly connected to two regional networks, say NYSERnet at 56kbps and JvNCnet

at  T1.   Normally the link to JvNCnet would be the best default.  Using RIP we

would configure our gateway to JvNCnet to advertise  a  default  route  with  a

lower  metric  than our gateway to NYSERnet.  However suppose something happens

within JvNCnet so that its access to the national networks is either cut off or

becomes  much worse (e.g. it might end up passing traffic through a backup link

to some other regional).  In that case it might be better to use NYSERnet.  The

typical  RIP  implementations  would  not  adapt  to  that  change, because the

boundary gateway is generating a default with a fixed metric.    With  IGRP  we

would  configure  our gateways so that the NSFnet national backbone was flagged

as a candidate default route.  In that case, we would choose the default  route

at  each gateway based on metric information that takes into account the entire

path back to the national backbone.  So the default would change to  take  into

account  events within our regional networks, assuming that they pass full IGRP

metric information to us.

5. Detailed Description

8

5.1 Overall Description

When a gateway is first turned on, its routing table is initialized.  This  may

be  done by an operator from a console terminal, or by reading information from

configuration files.  A description of each network connected to the gateway is

provided,  including  the  topological  delay along the link (i.e., how long it

takes a single bit to transverse the link) and the bandwidth of the link.

network 1      network 2         network 3     network 4

128.6.5         128.6.4           128.6.21      128.121

=====  =======================  ==========  ================

|    |          |        |    |      |    |         |

__|____|__   _____|____  __|____|__  __|____|____  ___|________

128.6.4.2    128.6.4.3   128.6.4.1   128.6.21.1    128.121.50.2

128.6.5.1                128.6.21.2  128.121.50.1

__________   __________  __________  ____________  ____________

gateway R   computer A   gateway S    gateway T    computer B

Fig 1.  A simple example network

For instance, in Fig. 1, gateway S would  be  told  that  it  is  connected  to

networks  2 and 3 via the corresponding interfaces.  Thus, initially, gateway 2

only knows that it can reach any destination computer in networks 2 and 3.  All

the  gateways  are  programmed  to  periodically  transmit to their neighboring

gateways the information that they have  been  initialized  with,  as  well  as

information  gathered  from  other  gateways.    Thus,  gateway S would receive

updates from gateways R and T and learn that it can reach computers in  network

1 through gateway R and computers in network 4 through gateway T. Since gateway

S sends its entire routing table, in the next cycle gateway T will  learn  that

it  can  get to network 1 through gateway S. It is easy to see that information

about every network in the system will eventually reach every  gateway  in  the

system, providing only that the network is fully connected.

________ network 1

|

gw A --nw2-- gw C

|          / |

|        /   |

nw3    nw4   nw5

|    /       |

|  /         |

gw B         gw D

____|____________|____  network 6

Fig 2.  Example of alternate paths

Each  gateway  computes a composite metric to determine the desirability of the

data paths to destination computers.  For instance, in Fig 2, for a destination

in  network  6,  gateway  A  would  compute metric functions for two paths, via

gateways B and C. Note that paths are defined simply by the next  hop.    There

are actually three possible routes from A to network 6:

- direct to B

- to C and then to B

- to C and then to D

However  gateway  A  need  not  choose  between the two routes involving C. The

routing table in A has a single entry representing the path to  C.  Its  metric

9

represents the best way of getting from C to the final destination.  If A sends

a packet to C, it is up to C to decide whether to use B or D.

The composite metric function computed for each data path is as follows:

[(K1 / Be) + (K2 * Dc)] r                            Eq. 1

Where:

r = fractional reliability

(% of transmissions that are sucessfully received at

the next hop.)

Dc = composite delay;

Be = effective bandwidth: unloaded bandwidth x

(1 - channel occupancy)

K1, K2 = constants.

In principle the composite delay, Dc, could be determined as follows:

Dc = Ds + Dcir + Dt                            Eq. 2

Where:

Ds = switching delay;

Dcir = circuit delay (propagation delay of 1 bit); and

Dt = transmission delay (no-load delay for a 1500 bit message).

However in practice a standard delay figure is used for each  type  of  network

technology.    E.g  there will be a standard delay figure for Ethernet, and for

serial lines at any particular bit rate.

Here is an example of how gateway A's routing table might look in the  case  of

Fig  2.    (Note that individual components of the metric vector are not shown,

for simplicity.)

interface   next gateway  metric

network 1     nw 1          none        directly connected

network 2     nw 2          none        directly connected

network 3     nw 3          none        directly connected

network 4     nw 2          C           1270

nw 3          B           1180

network 5     nw 2          C           1270

nw 3          B           2130

network 6     nw 2          C           2040

nw 3          B           1180

Fig 3.  An example routing table

The basic process of building up a routing table by exchanging information with

neighbors  is  described by the Bellman-Ford algorithm.  The algorithm has been

used in earlier protocols such as RIP (RFC 1058).  In order to deal  with  more

complex networks, IGRP adds three features to the basic Bellman-Ford algorithm:

1. Instead  of  a simple metric, a vector of metrics is used to characterize

paths.  A single composite  metric  can  be  computed  from  this  vector

10

according  to  equation  1.    Use  of  a  vector  allows  the gateway to

accomodate  different  types  of  service,  by  using  several  different

coefficients  in Eq. 1.  It also allows a more accurate representation of

the characteristics of the network than a single metric.

2. Instead of picking a single path with the  smallest  metric,  traffic  is

split  among  several  paths with metrics falling into a specified range.

This allows several routes to be used in parallel,  providing  a  greater

effective  bandwidth than any single route.  A variance V is specified by

the network administrator.  All paths with minimal composite metric M are

kept.    In addition, all paths whose metric is less than V x M are kept.

Traffic is distributed among multiple paths in inverse proportion to  the

composite metrics.

3. There  are  some problems with this concept of variance.  It is difficult

to come up with strategies that make use of variance values greater  than

1,  and  do  not also lead to packets looping.  In Cisco release 8.2, the

variance feature is not implemented.  (I am not sure in what release  the

feature  was  removed.)    The  effect  of  this  is  to set the variance

permanently to 1.

4. Several features are introduced to provide stability in situations  where

the topology is changing.  These features are intended to prevent routing

loops and "counting  to  infinity,"  which  have  characterized  previous

attempts  to  use Ford-type algorithms for this type of application.  The

primary stability features are "holddowns", "triggered  updates",  "split

horizon," and "poisoning".  These will be discussed in more detail below.

Traffic splitting (point 2) raises a rather subtle danger.  The variance  V  is

designed  to  allow  gateways  to  use parallel paths of different speeds.  For

example, there might be a 9600 bps line running in parallel with  a  19200  bps

line, for redundancy.  If the variance V is 1, only the best path will be used.

So the 9600 bps line will not be used if the 19200 bps line  has  a  reasonable

reliability.    (However if several paths are the same, the load will be shared

among them.)  By raising the variance, we can allow traffic to be split between

the  best  route and other routes that are nearly as good.  With a large enough

variance, traffic will be split between the two lines.  The danger is that with

a  large enough variance, paths become allowed that aren't just slower, but are

actually "in the wrong direction".  Thus there should be an additional rule  to

prevent  traffic  from  being  sent  "upstream": No traffic is sent along paths

whose remote composite metric (the composite metric calculated at the next hop)

is  greater  than  the  composite metric calculated at the gateway.  In general

system administrators are encouraged not to set the variance above 1 except  in

specific  situations  where  parallel paths need to be used.  In this case, the

variance is carefully set to provide the "right" results.

IGRP is intended to handle multiple "types of service," and multiple protocols.

Type of service is a specification in a data packet that modifies the way paths

are to be evaluted.  For example, the TCP/IP  protocol  allows  the  packet  to

specify  the  relative  importance  of  high  bandwidth,  low  delay,  or  high

reliability.  Generally,  interactive  applications  will  specify  low  delay,

whereas  bulk  transfer  applications  will  specify  high  bandwidth.    These

requirements determine the relative values of K1 and K2  that  are  appropriate

for  use in Eq. 1.  Each combination of specifications in the packet that is to

be supported is referred to as a "type of service".  For each type of  service,

a set of parameters K1 and K2 must be chosen.  A routing table is kept for each

type of service.  This is done because paths are selected and ordered according

to  the composite metric defined by Eq.  1.  This is different for each type of

service.  Information from all of these routing tables is combined  to  produce

the routing update messages exchanged by the gateways, as described in Fig. 7.

11

5.2 Stability Features

This  section  describes  holddowns,  triggered  updates,  split  horizon,  and

poisoning.  These features are designed to prevent  gateways  from  picking  up

erroneous  routes.    As  described  in  RFC 1058, this can happen when a route

becomes unusable, due to failure of a gateway or a network.  In principle,  the

adjacent  gateways  detect  failures.  They then send routing updates that show

the old route as unusable.  However it is possible for  updates  not  to  reach

some  parts  of  the  network  at  all,  or  to  be delayed in reaching certain

gateways.  A gateway that still believes the old route  is  good  can  continue

spreading  that  information, thus reentering the failed route into the system.

Eventually this information will propagate through the network and come back to

the gateway that reinjected it.  The result is a circular route.

In  fact  there  is  some  redundancy among the countermeasures.  In principle,

holddowns and triggered updates  should  be  sufficient  to  prevent  erroneous

routes  in  the  first  place.  However in practice, communications failures of

various kinds can cause them to be be insufficient.  Split  horizon  and  route

poisoning are intended to prevent routing loops in any case.

Normally,  new  routing  tables  are  sent to neighboring gateways on a regular

basis (every 90 seconds by default, although this can be adjusted by the system

administrator).    A  triggered  update  is  a  new  routing table that is sent

immediately, in response to some change.  The most important change is  removal

of  a  route.    This  can  happen  because  a  timeout has expired (probably a

neighboring gateway or line has gone down), or because an update  message  from

the  next  gateway in the path shows that the path is no longer usable.  When a

gateway G detects that a route is no  longer  usable,  it  triggers  an  update

immediately.    This  update  will  show that route as unusable.  Consider what

happens when this update reaches the neighboring gateways.  If  the  neighbor's

route  pointed  back to G, the neighbor must remove the route.  This causes the

neighbor to trigger an update, etc.  Thus a failure  will  trigger  a  wave  of

update  messages.    This  wave  will  propagate throughout that portion of the

network in which routes went through the failed gateway or network.

Triggered updates would be sufficent if we could guarantee  that  the  wave  of

updates  reached  every appropriate gateway immediately.  However there are two

problems.  First, packets containing the  update  message  can  be  dropped  or

corrupted  by  some  link  in the network.  Second, the triggered updates don't

happen instantaneously.  It is possible that a gateway that has not yet  gotten

the  triggered  update  will  issue  a  regular  update at just the wrong time,

causing the bad route to be reinserted in a neighbor that  had  already  gotten

the  triggered  update.    Holddowns are designed to get around these problems.

The holddown rule says that when a route is  removed,  no  new  route  will  be

accepted  for  the  same  destination  for some period of time.  This gives the

triggered updates time to get to all other gateways, so that we can be sure any

new  routes  we  get  aren't  just  some  gateway reinserting the old one.  The

holddown period must be long enough to allow for the wave of triggered  updates

to  go  throughout  the  network.    In addition, it should include a couple of

regular broadcast cycles, to handle dropped packets.  Consider what happens  if

one  of the triggered updates is dropped or corrupted.  The gateway that issued

that update will issue another update at the next regular update.    This  will

restart  the  wave  of  triggered  updates at neighbors that missed the initial

wave.

The combination of triggered updates and holddowns should be sufficient to  get

rid  of  expired  routes  and prevent them from being reinserted.  However some

additional precautions are worth doing anyway.    They  allow  for  very  lossy

networks,   and   networks  that  have  become  partitioned.    The  additional

precautions called for by IGRP are split horizon and route  poisoning.    Split

12

horizon  arises  from the observation that it never makes sense to send a route

back in the direction from which it came.  Consider the following situation:

network 1       network 2

-------------X-----------------X

gateway A         gateway B

Gateway A will tell B that it has a route to network 1.  When B  sends  updates

to A, there is never any reason for it to mention network 1.  Since A is closer

to 1, there is no reason for it to consider going via B. The split horizon rule

says  a separate update message should be generated for each neighbor (actually

each neighboring network).  The update for a given neighbor should omit  routes

that  point  to  that  neighbor.    This  rule  prevents loops between adjacent

gateways.  E.g.  suppose A's interface to network 1 fails.  Without  the  split

horizon  rule,  B  would be telling A that it can get to 1.  Since it no longer

has a real route, A might pick up that route.  In this case, A and B would both

have  routes  to  1.   But A would point to B and B would point to A. Of course

triggered updates and holddowns should prevent this from happening.  But  since

there's  no  reason  to  send information back to the place it came from, split

horizon is worth doing anyway.  In addition to its role  in  preventing  loops,

split horizon keeps down the size of update messages.

Split  horizon should prevent loops between adjacent gateways.  Route poisoning

is intended to break larger loops.  The rule is that when an update  shows  the

metric  for  an existing route to have increased sufficiently, there is a loop.

The route should be removed and put into holddown.  Currently the rule is  that

a route is removed if the composite metric increases more than a factor of 1.1.

It is not safe for just any increase in composite metric to trigger removal  of

the  route,  since  small  metric  changes  can occur due to changes in channel

occupancy or reliability.  So the factor of 1.1 is just a heuristic.  The exact

value  isn't  critical.    We  expect this rule only to be needed to break very

large loops, since small ones  will  be  prevented  by  triggered  updates  and

holddowns.

5.2.1 Disabling holddowns

As  of  release 8.2, Cisco's code provides an option to disable holddowns.  The

disadvantage of holddowns is that they delay adoption of a new  route  when  an

old route fails.  With default parameters, it can take several minutes before a

router adopts a new route after a change.  However for  the  reasons  explained

above, it is not safe simply to remove holddowns.  The result would be count to

infinity, as described in RFC 1058.  We conjecture, but cannot prove, that with

a  stronger  version  of route poisoning, holddowns are no longer needed to sto

count to infinity.  Thus disabling holddowns  enables  this  stronger  form  of

route  poisoning.    Note that split horizon and triggered updates are still in

effect.

The stronger form of route poisoning is based on a hop count.  If the hop count

for  a path increases, the route is removed.  This will obviously remove routes

that are still valid.  If something elsewhere in the network  changes  so  that

the  path  now  goes through one more gateway, the hop count will increase.  In

this case, the route is still valid.  However there is no completely  safe  way

to  distinguish  this  case  from  routing loops (count to infinity).  Thus the

safest approach is to remove the route whenever the hop count  increases.    If

the  route  is still legitimate, it will be reinstalled by the next update, and

that will cause a triggered update that will reinstall the route  elsewhere  in

the system.

3

In general, distance vector algorithms1adopt new routes easily.  The problem is

completely purging old ones from the system.    Thus  a  rule  that  is  overly

aggressive about removing suspicious routes should be safe.

5.3 Details of the Update Process

The  set  of  processes  described  in Fig. 4-8 are intended to handle a single

network protocol, e.g. TCP/IP,  DECnet,  or  the  ISO/OSI  protocol.    However

protocol  details  will be given only for TCP/IP.  A single gateway may process

data that follows more than one protocol.  Because each protocol has  different

addressing  structures  and packet formats, the computer code used to implement

Figs. 4-8 will generally be different for each protocol.  The process described

in  Fig.  4  will vary the most, as described in the detailed notes for Fig. 4.

The processes described in Fig. 5-8 will have the same general structure.   The

primary  difference from protocol to protocol will be the format of the routing

update packet, which  must  be  designed  to  be  compatible  with  a  specific

protocol.

Note  that  the definition of a destination may vary from protocol to protocol.

The method described here can be used  for  routing  to  individual  hosts,  to

networks,  or  for  more  complex  hierachical  address schemes.  Which type of

routing is used will depend upon the addressing structure of the protocol.  The

current  TCP/IP  implementation  supports  only  routing  to IP networks.  Thus

"destination" really means IP network or subnet number.  Subnet information  is

only kept for connected networks.

Figures  4-7 show pseudo-code for various pieces of the routing process used by

the gateways.  At the start of the program, acceptable protocols and parameters

describing each interface are entered.

The   gateway  will  only  handle  certain  protocols  which  are  listed.  Any

communication from a system using a protocol not on the list will  be  ignored.

The data inputs are

- the networks to which the gateway is connected

- the unloaded bandwidth of each networks

- topological delay of each network

- reliability of each network

- channel occupancy of each network

- MTU of each network

The  metric  function  for  each data path is then computed according to Eq. 1.

Note that the first three items are reasonably permanent.  They are a  function

of  the underlying network technology, and do not depend upon load.  They could

be set from a configuration file or by direct operator input.  Note  that  IGRP

does  not  use  measured  delay.  Both theory and experience suggest that it is

very difficult for  protocols  that  use  measured  delay  to  maintain  stable

routing.  There are two measured parameters: reliability and channel occupancy.

Reliability is based on error rates reported by the network interface  hardware

or firmware.

In  addition  these  inputs, the routing algorithm requires a value for several

routing  parameters.    This  includes  timer  values,  variance,  and  whether

holddowns  are  enabled.    This would normally be specified by a configuration

14

file or operator input.  (As of Cisco release 8.2, the variance is  permanently

set to 1.)

Once initial information is entered, operations in the gateway are triggered by

events - either the arrival of a data packet at one of the network  interfaces,

or  expiration  of a timer.  The processes described in Figs. 4-7 are triggered

as follows:

When a packet arrives, it is processed according to Fig 4.   This  results

in  the  packet  being sent out another interface, discarded, or

accepted for further processing.

When a packet is accepted by the gateway for  further  processing,  it  is

analyzed  in  a  protocol-specific fashion not described in this

specification.  If  the  packet  is  a  routing  update,  it  is

processed according to Fig. 5.

Figure  6 shows events triggered by a timer.  The timer is set to generate

an interrupt once per second.  When the  interrupt  occurs,  the

process shown in Fig. 6 is executed.

Figure  7 shows a routing update subroutine.  Calls to this subroutine are

shown in Figs 5 and 6.

In addition, Fig. 8 shows details of metric computations, referred  to  in

Fig. 5 and 7.

There  are  4  critical  time  constants  that  control  route  propagation and

expiration.  These time constants may  be  set  by  the  system  administrator.

However there are default values.  These time constants are

- broadcast  time  -  updates are broadcast by all gateways on all connected

interfaces this often.  The default is once every 90 sec.

- invalid time - if no update has been received for a given path within this

amount  of time, it is considered to have timed out.  It should be several

times the broadcast time, in order  to  allow  for  the  possibility  that

packets containing an update could be dropped by the network.  The default

is 3 times the broadcast time.

- hold time - when a destination has become unreachable (or the  metric  has

increased   enough   to   cause  poisoning),  the  destination  goes  into

"holddown".  During this state, no new path will be accepted for the  same

destination  for  this  amount  of time.  The hold time indicates how long

this state should last.  It should be several times  the  broadcast  time.

The  default  value  is  3  times  the  broadcast  time  plus 10 sec.  (As

described in section 5.2.1, it is possible to disable holddowns.)

- flush time - if no update has been received for a given destination within

this  amount  of time, the entry for it is removed from the routing table.

Note the difference between invalid  time  and  flush  time:    After  the

invalid  time  a path is timed out and removed.  If there are no remaining

paths to a destination, the destination is now unreachable.   However  the

database  entry for the destination remains.  It has to remain in order to

enforce the holddown.  After the flush time the database entry is  removed

from  the  table.  It should be somewhat longer than the invalid time plus

the holddown time.  The default is 7 times the broadcast time.

These figures presuppose the following major data structures.  A  separate  set

of  these  data  structures is kept for each protocol supported by the gateway.

Within each protocol, a separate set of data structures is kept for  each  type

15

of service to be supported.

For  each destination known to the system, there is a (possibly null) list

of paths to the destination, a holddown expiration time,  and  a

last  update time.  The last update time indicates the last time

any path for this destination was included  in  an  update  from

another gateway.  Note that there are also update times kept for

each path.  Holddowns are discussed in section 5.2.    When  the

last  path  to  a destination is removed, the destination is put

into  holddown,  unless  holddowns  are  disabled  (See  section

5.2.1).    The  holddown  expiration  time indicates the time at

which the holddown expires.    The  fact  that  it  is  non-zero

indicates that the destination is in holddown.  In order to save

calculation time, it is also a good idea to keep a "best metric"

for  each  destination.    This  is  simply  the  minimum of the

composite metrics for all the paths to the destination.

For each path to a destination, there is the address of the  next  hop  in

the  path,  the  interface  to  be  used,  a  vector  of metrics

characterizing the path, including topological delay, bandwidth,

reliability,  and  channel occupancy.  Other information is also

associated with each path, including hop count, MTU,  source  of

information, the remote composite metric, and a composite metric

calculated from these numbers according to equation 1.  There is

also  a  last  update time.  The source of information indicates

where the most recent update  for  that  path  came  from.    In

practice  this  is the same as the address of the next hop.  The

last update time is simply the time at  which  the  most  recent

update  arrived  for  this path.  It is used to expire timed-out

paths.

Note that an IGRP update message has three portions: interior, system  (meaning

"this autonomous system" but not interior), and exterior.  The interior section

is for routes to subnets.  Not  all  subnet  information  is  included.    Only

subnets  of  one network are included.  This is the network associated with the

address to which the update is being sent.  Normally updates are  broadcast  on

each  interface,  so this is simply the network on which the broadcast is being

sent.  (Other cases arise for responses to an IGRP request and point  to  point

IGRP.)    Major  networks (i.e. non-subnets) are put into the system portion of

the update message unless they are specifically flagged as exterior.

A network will be flagged as exterior if it was learned  from  another  gateway

and  the  information  arrived  in  the exterior portion of the update message.

cisco's implementation also allows the system administrator to declare specific

networks  as  exterior.    Exterior  routes  are also referred to as "candidate

default".  They are routes that go to or through gateways that  are  considered

to  be appropriate as defaults, to be used when there is no explicit route to a

destination.  For example at Rutgers we configure  the  gateway  that  connects

Rutgers  to  our  regional  network  so  that  it flags the route to the NSFnet

backbone as exterior.   cisco's  implementation  chooses  a  default  route  by

picking that exterior route with the smallest metric.

The following sections are intended to clarify certain portions of Fig. 4-8.

16

5.4 Packet Routing

Fig  4  describes  overall processing of input packets.  This is used simply to

clarify terminology.  Obviously this is not a complete description of  what  an

IP gateway does.

This process uses the list of supported protocols and the information about the

interfaces entered when the gateway is initialized.    Details  of  the  packet

processing  depend upon the protocol used by the packet.  This is determined in

Step A. Step A is the only portion of Fig. 4 which is shared by all  protocols.

Once  the  protocol  type is known, the implementation of Fig. 4 appropriate to

the protocol type is used.  Details of the packet contents are described by the

specifications  of  the  protocol.   The specifications of a protocol include a

procedure for  determining  the  destination  of  a  packet,  a  procedure  for

comparing the destination with the gateway's own addresses to determine whether

the gateway itself is the destination, a procedure for  determining  whether  a

packet  is a broadcast, and a procedure for determining whether the destination

is part of a specified network.  These procedures are used in steps B and C  of

Fig. 4.  The test in step D requires a search of the destinations listed in the

routing table.  The test is satisfied if there is an entry in the routing table

for  the  destination, and that destination has associated with it at least one

usable path.  Note that the destination and path data used in this and the next

step  are  maintained separately for each type of service supported.  Thus this

step begins by determining the type of service specified  by  the  packet,  and

selecting the corresponding set of data structures to use for this and the next

step.

A path is usable for the purposes of steps D and  E  if  its  remote  composite

metric is less than its composite metric.  A path whose remote composite metric

is greater than its composite metric is a path whose next hop is "farther away"

from  the  destination,  as  measured by the metric.  This is referred to as an

"upstream path." Normally one would  expect  that  the  use  of  metrics  would

prevent  upstream  paths from being chosen.  It is easy to see that an upstream

path can never be the best one.  However if a large variance is allowed,  paths

other than the best one can be used.  Some of those could be upstream.

Step  E  computes  the path to use.  Paths whose remote composite metric is not

less than their composite metrics are not considered.  If more than one path is

acceptable,  such paths are used in a weighted form of round-robin alternation.

The frequency with which a path  is  used  is  inversely  proportional  to  its

composite metric.

5.5 Reception Of Routing Updates

Fig  5 describes the processing of a routing update received from a neighboring

gateway.  Such updates consist of a  list  of  entries,  each  of  which  gives

information  for  a  single  destination.    More  than  one entry for the same

destination can occur in a single routing update, to accomodate multiple  types

of  service.   Each of these entries is processed individually, as described in

Fig. 5.  If an entry is in the exterior section of  the  update,  the  exterior

flag  will  be  set  for  the  destination  if  it is added as a result of this

process.

The entire process described in figure 5 must be repeated once for each type of

service  supported  by  the  gateway,  using  the  set  of  destination  / path

information associated with that type  of  service.    This  is  shown  in  the

outermost loop in Fig. 5.  The entire routing update must be processed once for

17

each type of service.  (Note that the current implementation of IGRP  does  not

support  multiple  types  of  service.    So the outermost loop is not actually

implemented.)

In step A, basic acceptability tests are done on the path.  This should include

reasonableness  tests  for  the  destination.    Impossible ("Martian") network

numbers should be rejected.  (See RFC 1009 and RFC 1122 for details.)   Updates

are  also  rejected  if  the destination they refer to is in holddown, i.e. the

holddown expiration time is non-zero and later than the current time.

In steps B the routing table is searched to see whether this entry describes  a

path  that  is  already  known.   A path in the routing table is defined by the

destination with which it is associated, the next hop listed  as  part  of  the

path,  the output interface to be used for the path, and the information source

(the address from which the update came -- in practice normally the same as the

next hop).  The entry from the update packet describes a path whose destination

is listed in the entry, whose output interface is the interface that the update

came  in,  and  whose  next  hop  and information source are the address of the

gateway that sent the update (the "source" S).

In steps H and T, the update process described in Fig. 7 is  scheduled.    This

process will actually run after the entire process described in Fig. 5 has been

finished.  That is, the update process described in Fig.  7  will  only  happen

once,  even if it is triggered several times during the processing described in

Fig. 5.  Furthermore, precautions must be taken  to  keep  updates  from  being

issued too often, if the network is changing rapidly.

Steps K is done if the destination described by the current entry in the update

packet already exists in the routing table.    K  compares  the  new  composite

metric  computed  from data in the update packet with the best composite metric

for the destination.  Note that the best composite metric is not recomputed  at

this  time.    So if the path being considered is already in the routing table,

this test may compare new and old metrics for the same path.

Step L is performed for the  paths  that  are  worse  than  the  existing  best

composite  metric.    This includes both new paths that are worse than existing

ones and existing paths whose composite metric has increased.    Step  L  tests

whether  the  new  path is acceptable.  Note that this test implements both the

test for whether a new path is good enough to keep, and route  poisoning.    In

order  to  be  acceptable,  the  delay value must not be the special value that

indicates an unreachable destination (for the current  IP  implementation,  all

ones  in  a 24 bit field), and the composite metric (calculated as specified in

Fig. 8) must be acceptable.  To  determine  whether  the  composite  metric  is

acceptable,  compare  it  with  the composite metrics of all other paths to the

destination.  Let M be the minimum of these.  The new path is acceptable if  it

is < V x M, where V is the variance set when the gateway was initialized.  If V

= 1 (which is always true as of Cisco release 8.2), then  a  metric  any  worse

than  the  existing one is not acceptable.  There is one exception to this:  If

the path already exists and is the only path to the destination, the path  will

be  retained  if  the  metric  has  not  increased  by  more than 10% (or where

holddowns are disabled, if the hop count has not increased).

Step V is done when the new information for a path indicates that the composite

metric  will be decreased.  The composite metrics of all paths to destination D

are compared.  In this comparison, the new composite  metric  for  P  is  used,

rather than the one appearing in routing table.  The minimum composite metric M

is calculated.  Then all paths to D are  examined  again.    If  the  composite

metric  for any path > M x V, that path is removed.  V is the variance, entered

when the gateway was initialized.  (As of Cisco release 8.2,  the  variance  is

permanently set to 1.)

18

5.6 Periodic Processing

The  process  described  in  Figure  6 is triggered once a second.  It examines

various timers in the routing table, to see if any has expired.   These  timers

are described above.

In step U, the process described in Fig. 7 is activated.

Steps R and S are necessary because the composite metrics stored in the routing

table depend upon the channel occupancy, which  changes  over  time,  based  on

measurements.    Periodically  the  channel  occupancy is recalculated, using a

moving  average  of  measured  traffic  through  the   interface.      If   the

newly-calculated  value  differs  from  the existing one, all composite metrics

involving that interface must be adjusted.  Every path  shown  in  the  routing

table  is  examined.    Any  path  whose  next  hop  uses interface "I" has its

composite metric recalculated.  This is done in accordance with eq. 1, using as

the  channel occupancy the maximum of the value stored in routing table as part

of the path's metric,  and  the  newly  calculated  channel  occupancy  of  the

interface.

5.7 Generating Update Messages

Figure  7  describes  how  the  gateway generates update messages to be sent to

other gateways.  A separate message is generated  for  each  network  interface

attached  to the gateway.  That message is then sent to all other gateways that

are reachable through the interface (step  J).    Generally  this  is  done  by

sending  the  message  as  a  broadcast.  However, if the network technology or

protocol does not allow broadcasts, it may be necessary  to  send  the  message

individually to each gateway.

In  general, the message is built up by adding an entry for each destination in

the routing table, in step G. Note that the destination / path data  associated

with  each  type  of  service  must be used.  In the worst case, a new entry is

added to the update for each destination for each type of  service.    However,

before  adding  an  entry  to the update message in step G, the entries already

added are scanned.  If the new entry is already present in the update  message,

it  is  not  added  again.    A  new  entry duplicates an existing one when the

destinations and next hop gateways are the same.

For the sake of simplicity,  the  pseudo-code  omits  one  thing:  IGRP  update

messages  have  three  parts:  interior,  system, and exterior.  Thus there are

actually three loops over destinations.  The first includes only subnets of the

network  to  which  the  update  is  being sent.  The second includes all major

networks (i.e. non-subnets) that are  not  flagged  as  exterior.    The  third

includes all major networks that are flagged as exterior.

Step  E implements the split horizon test.  In the normal case, this test fails

for routes whose best path goes out the same interface that the update is being

sent  out.  However if the update is being sent to a specific destination (e.g.

in response to an IGRP request from another gateway, or as part  of  "point  to

point  IGRP"),  split  horizon fails only if the best path originally came from

that destination (its "information source" is the same as the destination)  and

its output interface is the same as the one the request came in from.

19

5.8 Computing Metric Information

Fig.  8  describes how the metric information is processed from update messages

received by the gateway, and how it is generated for update messages being sent

by  the  gateway.    Note that the entry is based on one particular path to the

destination.  If there is more than one path to the destination, a  path  whose

composite  metric  is minimum is chosen.  If more than one path has the minimum

composite metric, an arbitrary tie-breaking rule is used.  (For most protocols,

this will be based on the address of the next hop gateway.).

20

Fig 4. Processing incoming packets

Data packet arrives using interface I

A      Determine protocol used by packet

If protocol is not supported

then discard packet

B      If destination address matches any of gateway's addresses

or the broadcast address

then process packet in protocol-specific way

C      If destination is on a directly-connected network

then send packet direct to the destination, using

the encapsulation appropriate to the protocol

and link type

D      If there are no paths to the destination in the routing

table, or all paths are upstream

then send protocol-specific error message and

discard the packet

E      Choose the next path to use.  If there are more than

one, alternate round-robin with frequency proportional

to inverse of composite metric.

Get next hop from path chosen in previous step.

Send packet to next hop, using encapsulation appropriate

to protocol and data link type.

21

Fig 5.  Processing incoming routing updates

Routing update arrives from source S

For each type of service supported by gateway

Use routing data associated with this type of service

For each destination D shown in update

A              If D is unacceptable or in holddown

then ignore this entry and continue loop

with next destination D

B              Compute metrics for path P to D via S  (see Fig 8)

If destination D is not already in the routing table

then Begin

Add path P to the routing table, setting last

update times for P and D to current time.

H                  Trigger an update

Set composite metric for D and P to new composite

metric computed in step B.

End

22

Else begin  (dest. D is already in routing table)

K                  Compare the new composite metric for P with best

existing metric for D.

New > old:

L                        If D is shown as unreachable in the update,

or holddowns are enabled and

the new composite metric >

(the existing metric for D) * V

[use 1.1 instead of V if V = 1,

as it is as of Cisco release 8.2]

O                        or holddowns are disabled and

P has a new hop count > old hop count

then Begin

Remove P from routing table if present

If P was the last route to D

then Unless holddowns are disabled

Set holddown time for D to

current time + holddown time

T                                    and Trigger an update

End

else Begin

Compute new best composite metric for D

Put the new metric information into the

entry for P in the routing table

Add path P to the routing table if it

was not present.

Set last update times for P and D to

current time.

End

New <= old:

V                        Set composite metric for D and P to new

composite metric computed in step B.

If any other paths to D are now outside the

variance, remove them.

Put the new metric information into the

entry for P in the routing table

Set last update times for P and D to

current time.

End

End of for

End of for

23

Fig 6.  Periodic processing

Process is activated by regular clock, e.g. once per second

For each path P in the routing table (except directly

connected interfaces)

If current time < P's last update time + invalid time

then continue with the next path P

Remove P from routing table

If P was the last route to D

then Set metric for D to inaccessible

Unless holddowns are disabled,

Start holddown timer for D and

Trigger an update

else Recompute the best metric for D

End of for

For each destination D in the routing table

If D's metric is inaccessible

then Begin

Clear all paths to D

If current time >= D's last update time + flush time

then Remove entry for D

End

End of for

For each network interface I attached to the gateway

R          Recompute channel occupancy and error rate

S          If channel occupancy or error rate has changed,

then recompute metrics

End of for

At intevals of broadcast time

U          Trigger update

24

Fig 7.  Generate update

Process is caused by "trigger update"

For each network interface I attached to the gateway

Create empty update message

For each type of service S supported

Use path/destination data for S

For each destination D

E                  If any paths to D have a next hop reached through I

then continue with the next destination

If any paths to D with minimal composite metric are

already in the update message

then continue with the next destination

G                  Create an entry for D in the update message, using

metric information from a path with minimal

composite metric (see Fig. 8)

End of for

End of for

J          If there are any entries in the update message

then send it out interface I

End of for

25

Fig 8.  Details of metric computations

This  section describes the procedure for computing metrics and hop counts from

an arriving routing update.

The input to this function is the entry for a specific destination in a routing

update  packet.  The output is a vector of metrics which can be used to compute

the composite metric, and a hop count.  If this path is added  to  the  routing

table,  the  entire  vector  of metrics is entered in the table.  The interface

parameters used in the following definitions are those set when the gateway was

initialized, for the interface on which the routing update arrived, except that

the channel occupancy and reliability are based on a moving average of measured

traffic through the interface.

delay = delay from packet + interface topological delay

bandwidth = max (bandwidth from packet, interface bandwidth)

reliability = min (reliability from packet, interface reliability)

channel occupancy = max (channel occupancy from packet,

interface channel occupancy)

(Max  is  used  for bandwidth because the bandwidth metric is stored in inverse

form.  Conceptually, we want the minimum bandwidth.)  Note  that  the  original

channel  occupancy  from  the  packet must be saved, since it will be needed to

recompute the  effective  channel  occupancy  whenever  the  interface  channel

occupancy changes.

The  following  are  not  part  of  the metric vector, but are also kept in the

routing table as characteristics of the path:

hop count = hop count from packet

MTU = min (MTU from packet, interface MTU)

remote composite metric - calculated from Eq. 1 using  the  metric  values

from  the packet.  That is, the metric components are those from

the packet, and are not updated as shown above.  Obviously  this

must be calculated before the adjustments shown above are done.

composite metric- calculated from Eq. 1 using the metric values calculated

as described in this section

26

This section describes the procedure for computing metrics and  hop  count  for

routing updates to be sent:

This function determines the metric information and hop count to be put into an

outgoing update packet.  It is based on a specific path to  a  destination,  if

there  are  any  usable  paths.    If  there are no paths, or the paths are all

upstream, the destination is called "inaccesible".

If destination is inaccessible, this is  indicated  by  using  a  specific

value  in  the  delay  field.  This value is chosen to be larger

than the largest valid delay.  For the IP implementation this is

all ones in a 24-bit field.

If  destination  is  directly reachable through one of the interfaces, use

the delay, bandwidth, reliability, and channel occupancy of  the

interface.  Set hop count to 0.

Otherwise,  use  the  vector  of  metrics  associated with the path in the

routing table.  Add one to the hop count from the  path  in  the

routing table.

6. Details of the IP implementation

This  section  gives  a brief description of the packet formats used by cisco's

IGRP.  IGRP is sent using IP datagrams with IP protocol 9 (IGP).    The  packet

begins with a header.  It starts immediately after the IP header.

unsigned version: 4;    /* protocol version number */

unsigned opcode: 4;     /* opcode */

uchar edition;          /* edition number */

ushort asystem;         /* autonomous system number */

ushort ninterior;       /* number of subnets in local net */

ushort nsystem;         /* number of networks in AS */

ushort nexterior;       /* number of networks outside AS */

ushort checksum;        /* checksum of IGRP header and data */

For update messages, routing information follows immediately after the header.

The  version  number  is currently 1.  Packets having other version numbers are

ignored.

The opcode is either

1         update

2         request

This indicates the type of message.  The format of the two message  types  will

be given below.

Edition  is  a serial number which is incremented whenever there is a change in

the routing table.  (This is done in those conditions in which the  pseudo-code

above says to trigger a routing update.)  The edition number allows gateways to

avoid processing updates containing information that they  have  already  seen.

(This  is  not currently implemented.  That is, the edition number is generated

correctly, but it is ignored on input.  Because it is possible for  packets  to

be  dropped,  it  is  not  clear that the edition number is sufficient to avoid

duplicate processing.  It would be necessary to  make  sure  that  all  of  the

27

packets associated with the edition had been processed.)

Asystem  is  the autonomous system number.  In cisco's implementation a gateway

can participate in more than one autonomous system.  Each such system runs  its

own  IGRP protocol.  Conceptually, there are completely separate routing tables

for each autonomous system.  Routes that arrive via IGRP  from  one  autonomous

system  are sent only in updates for that AS.  This field allows the gateway to

select which set of routing tables to use for processing this message.  If  the

gateway receives an IGRP message for an AS that it is not configured for, it is

ignored.  In fact cisco's implementation allows information to be "leaked" from

one  AS  to  another.  However I regard that as an administrative tool, and not

part of the protocol.

Ninterior, nsystem, and nexterior indicate the number of entries in each of the

three  sections  of update messages.  These sections have been described above.

There is no other demarcation  between  the  sections.    The  first  ninterior

entries are taken to be interior, the next nsystem entries as being system, and

the final nexterior as exterior.

Checksum is an IP checksum, computed using the same checksum algorithm as a UDP

checksum.    The  checksum  is  computed  on  the  IGRP  header and any routing

information that follows it.  The checksum field is set to zero when  computing

the  checksum.    The checksum does not include the IP header, nor is there any

virtual header as in UDP and TCP.

6.1 Requests

An IGRP request asks the recipient to send its  routing  table.    The  request

message  has  only  a header.  Only the version, opcode, and asystem fields are

used.  All other fields are zero.  The recipient is expected to send  a  normal

IGRP update message to the requester.

6.2 Updates

An  IGRP  update  message  contains  a  header, followed immediately by routing

entries.  As many routing entries are included as will  fit  into  a  1500-byte

datagram  (including  IP  header).    With current structure declarations, this

allows up to 104 entries.  If more entries are needed, several update  messages

are  sent.  Since update messages are simply processed entry by entry, there is

no  advantage  to  using  a  single  fragmented  message  rather  than  several

independent ones.

Here is the structure of a routing entry:

uchar number[3];     /* 3 significant octets of IP address */

uchar delay[3];      /* delay, in tens of microseconds */

uchar bandwidth[3];  /* bandwidth, in units of 1 Kbit/sec */

uchar mtu[2];        /* MTU, in octets */

uchar reliability;   /* percent packets successfully tx/rx */

uchar load;          /* percent of channel occupied */

uchar hopcount;      /* hop count */

The  fields  defined  uchar[2]  and  uchar[3]  are  simply 16 and 24 bit binary

integers, in normal IP network order.

28

Number defines the destination being described.  It is an IP address.  To  save

space,  only  the  first  3  bytes  of  the IP address are given, except in the

interior section.  In the interior section, the last 3 bytes are  given.    For

system  and  exterior routes, no subnets are possible, so the low-order byte is

always zero.  Interior routes are always subnets of a  known  network,  so  the

first byte of that network number is supplied.

Delay is in units of 10 microseconds.  This gives a range of 10 microseconds to

168 seconds, which seems sufficient.  A delay of all ones  indicates  that  the

network is unreachable.

Bandwidth  is  inverse  bandwidth in bits per sec scaled by a factor of 1.0e10.

The range is from a 1200 bps line to 10 Gbps.  (That is, if the bandwidth is  N

kbps, the number used is 10000000 / N.)

MTU is in bytes.

Reliability is given as a fraction of 255.  That is, 255 is 100%.

Load is given as a fraction of 255.

Hopcount is a simple count.

Because of the somewhat wierd units used for bandwidth and delay, some examples

seem in order.  These are the default values used for several common media.

Delay           Bandwidth

Satellite    200,000 (2 sec)           20 (500 Mbit)

Ethernet         100 (1 ms)         1,000

1.544 Mbit      2000 (20 ms)        6,476

64 Kbit         2000              156,250

56 Kbit         2000              178,571

10 Kbit         2000            1,000,000

1 Kbit          2000           10,000,000

6.3 Metric computations

Here is a description of the way the composite metric is actually  computed  in

cisco version 8.0(3).

metric = [K1*bandwidth + (K2*bandwidth)/(256 - load) + K3*delay] *

[K5/(reliability + K4)]

If K5 == 0, the reliability term is not included.

The default version of IGRP has K1 == K3 == 1, K2 == K4 == K5 == 0

29