Using TIGA in the 640x480x16 mode

---------------------------------

D. C. Rhodes

Technical Support Manager

Hercules Computer Technology

7/25/90

Writing applications that use the Graphics Station Card

640x480x16-bit mode is not very different than writing for

the 8-bit modes.  In fact, the only consideration to be aware

of is that in the 16-bit mode, there is no palette.  The 32,768

colors available for display may not be changed.

Because of this fact, functions relating to the palette

(get_nearest_color, get_palet, get_palet_entry, init_palet,

set_palet and set_palet_entry) have no function and should not

be used.  However, for the sake of writing portable code, calling

these functions while in the 16-bit mode will not have any

adverse effects.

Colors are still set with set_fcolor() and set_bcolor().

In the 8-bit TIGA modes, these routines are called with a value

of 0-255 which is an index into a 24-bit palette.  In the 16-bit

mode, a 2-byte value corresponding to the absolute 15-bit color

is passed.  The structure of these bytes is as follows:

* R R R R R G G | G G G B B B B B

7 6 5 4 3 2 1 0 | 7 6 5 4 3 2 1 0

MSB               LSB

* The rightmost bit of the LSB is unused at the present

time.

An example: To express pure green, pass the value

0x0000001111100000, or 32 + 64 + 128 + 256 + 512, or 992.

We can define the pure colors as follows:

#define	RED_16		31744

#define	GREEN_16	992

#define	BLUE_16		31

#define	WHITE_16	32767

Note that for black, we can continue to use the 8-bit

definition for BLACK, which is 0.

To make the special color considerations of the 16-bit

mode nearly transparent, we simply need a routine which accepts

as arguments three (5 bit or less) numbers for the red, green

and blue components, and which returns a long value which is

the three components scrunched together.  I have written it

as a macro:

#define	col16(r,g,b)	(r << 10) + (g << 5) + b

Each argument is an integer between 0 and 31 inclusive,

where 0 = minimum saturation and 31 = maximum saturation.  To

set the foreground color to a bluish gray, you would call

the following:

#define	BLUEGRAY	col16(15,15,31)

set_fcolor(bluegray);

or, just:

set_fcolor( col16(15,15,31) );

Note: If you call the col16() macro with identifiers rather

than values, the identifiers should be delcared as ints or longs,

rather than as chars.

The rest should be smooth sailing!

Here is a sample program which demonstrates the points

mentioned in this document.  The program first places a patriotic

pattern on the screen, then displays a range of 640 colors from

the 32,768 colors available.

#include <typedefs.h>

#include <tiga.h>

#include <extend.h>

CONFIG config;

#define	col16(r,g,b)	(r << 10) + (g << 5) + b

#define	RED_16		31744

#define	GREEN_16	992

#define	BLUE_16		31

#define	WHITE_16	32767

#define	BLUEGRAY	col16(15,15,31)

main()

{

int ip_result;

int i;

/*

*

* First, set the video mode.

*

*/

if (!(set_videomode(TIGA, INIT_GLOBALS | CLR_SCREEN) )) {

printf("Error: set_videomode() has failed.\n");

exit(-1);

}

/*

*

* Next, install the extended primitives.

*

* See page 3-99 of the TIGA-340 User's Guide for

* an explanation of the error result codes.

*

*/

if (!function_implemented(DRAW_LINE)) {

if ( (ip_result = install_primitives()) < 0) {

set_videomode(PREVIOUS, INIT);

printf("Error: install_primitives() has failed\n");

printf("with an error code of %d.\n",ip_result);

exit(-1);

}

}

/* 640x480x16 is mode 1. */

set_config(1,1);

clear_page(BLUEGRAY);

set_fcolor(RED_16);

fill_rect(150,300,95,90);

set_fcolor(WHITE_16);

fill_rect(150,300,245,90);

set_fcolor(BLUE_16);

fill_rect(150,300,395,90);

getch();

for (i = 0; i < 640; i++) {

set_fcolor( i * (32768/640));

draw_line(i,0,i,480);

}

getch();

set_videomode(VGA, INIT);

}