Programming the 24-bit Graphics Mode via TIGA

Dylan Rhodes, Hercules Tech Support

7/11/90

With the 7/13 release of the GB1024 Utilities Disk, access

to the 512x480 mode via TIGA has been enabled.  This document provides

an introduction to 24-bit TIGA programming.

As released, none of the extended primitives work in 24-bit

mode (most were designed to work at 16 bits or less, as that is TIGA 1.1's

theoretical capacity).  Core primitive support is limited to the following

routines:

Mode Setting Calls:

set_videomode()

get_videomode()

set_config()

get_config()

Memory Management Calls:

gsp2gsp()

gsp2host()

gsp_calloc()

gsp_execute()

gsp_free()

gsp_malloc()

gsp_maxheap()

gsp_minit()

gsp_realloc()

host2gsp()

Putting the Card into 24-bit Mode

---------------------------------

To place the card in 24-bit mode via TIGA, use the set_videomode()

procedure as usual.  e.g.:

set_videomode(TIGA, INIT_GLOBALS | CLR_SCREEN);

Next, to set the resolution to 512x480x24, use video mode 5:

set_config(5,1);

Next, the memory heap must be reinitialized whenever the mode is changed:

gsp_minit(-1);

Putting Graphics On The Screen

------------------------------

In the 24-bit mode, the buffer is contiguous, with four bytes per pixel.  The

byte order is:

0:	Green Component (0-255)

1:	Red Component (0-255)

2:	Blue Component (0-255)

3:	Unused

Each line is (512 * 4) = 2048 bytes wide.

A pixel's x,y address may be calculated as (2048 * y) + (4 * x).

The best way to move data to the screen is with the host2gsp() procedure.

host2gsp() moves a chunk of memory from DOS memory to the display buffer

memory.  Conversely, gsp2host() moves data from the display buffer memory

to a specified DOS memory address.  As this is the extent of graphics

capabilities of the 24-bit mode at this time, the mode is naturally suited

for non-interactive tasks such as displaying of video images.

An example:  plot24()

This routine accepts an x,y value, and the RGB components of the pixel to

be plotted.  This routine is probably slow and unoptimized, but it

serves as a good example of pixel addressing.

plot24( x, y, r, g, b)

int x, y;

unsigned char r, g, b;

{

char far *hpt;

long addr;

hpt = (char far *) malloc(3);

hpt[0] = g;

hpt[1] = r;

hpt[2] = b;

host2gsp(hpt, (0x4000L * y) + (x * 32L), 0x3L, 0);

free(hpt);

}

Here's another routine which clears the screen to a specified color.

It is not very efficient - it simply copies one scan line to the screen

and then copies that scan line another 480 times.  The simple task

of optimizing this code is left up to the reader.

clear(r, g, b)

unsigned r, g, b;

{

char far *hpt;

long addr;

int i;

hpt = (char far *) malloc(2048);

for (i = 0; i < 2048; i+=4) {

hpt[i] = g;

hpt[i+1] = r;

hpt[i+2] = b;

}

addr = 0x4000L;

host2gsp(hpt, 0L, 2048, 0);

for (i = 0; i < 480; i++) {

gsp2gsp(0L, addr, 2048L);

addr += 0x4000L;

}

free(hpt);

}

The only logical way to use the 24-bit mode for line-drawing, shapes,

fills, etc. is to compile these routines with the TI 34010 C Compiler

and upload them as an RLM or ALM.  As this is not a viable procedure

for many users, Hercules is developing a GSP-based library of drawing

primitives for use in the 24-bit mode.