WESTERN DIGITAL

"PLUS"-FAMILY LAN ADAPTER BOARD DRIVER

FOR USE WITH 3COM'S 3+ NETWORK OPERATING SYSTEM

INSTALLATION GUIDE         May 12, 1988

This driver is for use with the WD8003-Family LAN Adapters: StarCard

PLUS (tm), StarLink PLUS (tm), and EtherCard PLUS (tm).

This document contains the information about how to install a Western

Digital WD8003-family LAN adapter board and driver in 3Com's 3+ (*)

network.

1. Jumper the adapter according to your system configuration.

The WD.SYS Device Driver is installed at PC boot time according to

the command line specified in the DOS CONFIG.SYS file (described below).

The user must verify that the jumper settings on the adapter board

correspond to the software settings given on the device driver command

line.

Jumper settings for some adapter boards are described in the

JUMPERS.DOC file.  For more information, refer to the Western Digital

"User Installation Guide" that comes with each adapter board.

2. Insert the adapter in your PC.

3. Power up your PC.

4. Install the 3Com 3+ network software as specified in the 3Com

Installation and Configuration Guide.

5. After the installation is complete, copy the Western Digital driver

WD.SYS from this diskette to the root directory of your system drive

with a new name called ETH.SYS.  For example,

CD A:\3PLUS

COPY A:WD.SYS C:\ETH.SYS

If 3+NetConnect (*) has been installed and the Western Digital board

is being used as a remote (secondary) network adapter, copy the

secondary driver from this disk to the root directory.  For example,

COPY A:2WD.SYS C:\2ETH.SYS

6. If you changed the LAN adapter board's default parameters in step

1, you need to modify the DEVICE=[2]ETH.SYS command line in your

CONFIG.SYS file to reflect the jumper selections of step 1.  Use the

format:

DEVICE=ETH.SYS irq base page

DEVICE=2ETH.SYS irq base page

where,

irq  - IRQ level to receive adapter board interrupts         (3)

base - I/O base address (in hexadecimal)                   (280)

page - 4-digit memory segment byte address of the 8 Kbyte

shared buffer memory (not jumpered on the board)   (C400)

The factory jumper settings for the WD8003-family adapter boards and

the WD.SYS default buffer memory base address are given in parentheses.

If these default values conflict with other adapter boards installed

in your system, you may need to change the values used by the LAN

adapter. For example, if you want to change the IRQ level to 4 and

the buffer memory base address to D0000 (segment address D000), then

you'll have to change the line

"DEVICE=ETH.SYS" to "DEVICE=ETH.SYS 4 280 D000"

before rebooting the system.

PLEASE NOTE:  There are a number of PC boards that can conflict with

various possible shared memory addresses.  For example, the VGA and

EGA graphics adapters seem to use the area of memory in the range of

C000 through D400, depending on the type of adapter.  We have found

D000 to be a good address for users of EGA adapters;  D400 or DE00

work well for systems with the VGA adapters.  If you are seeing any

problems in getting the system to function, try moving the shared

memory base address.

7. Reboot the system.

(*) 3Com and 3+ are registered trademarks and 3+NetConnect is a

trademark of 3Com Corporation.

EtherCard PLUS, StarCard PLUS, and StarLink PLUS are trademarks

of Western Digital Corporation.